PREFINAL ACTIVITY

A.Y. 2025-2026 (1st Semester)

ACT. ASSIGNMENT 3.1

Name: Lovely Jane Alonso                                  Date: 10-21-2025

Yr. & Program: 2B-BSIT                                    Score: ________

How to Configure Router

A. Accessing the Router Settings

- Connect your computer to the router using Wi-Fi or a LAN cable.

- Open a web browser and type 192.168.0.1 or 192.168.1.1 in the address bar.

- Log in using the default username and password — both are usually admin.


B. Changing the Router’s IP Address

- Go to the Network → LAN section.

- Edit the IP Address (example: change 192.168.0.1 to 192.168.10.1).

- Click Save — the router will restart.

- Reconnect using the new IP address.

C. Setting Up DHCP

- Go to DHCP → DHCP Settings.

- Turn DHCP Server to Enabled.

- Set the Start IP Address and End IP Address ( example: 192.168.10.100 to 192.168.10.199).

- Use 192.168.10.1 as the Default Gateway.

- Click Save.

D. Changing Wi-Fi Name (SSID) and Password

- Go to Wireless → Basic Settings.

- Enter your new Wi-Fi name (SSID), example: sccwifi, then click Save.

- Go to Wireless → Security.

- Choose WPA/WPA2-Personal for better security.

- In the Password box, type your new Wi-Fi password (for example: qwe123!@#).

- Click Save, then reconnect using your new Wi-Fi details.

E. Final Step

- Restart the router to apply all changes.

- Check if your new Wi-Fi name shows up and confirm you can connect to the internet.